# Confined Space Entry – Standard Operating Procedure

Document No: SOP-SAF-014   Revision: 4   Effective: 01-Mar-2025
Owner: Plant Safety Department   Approver: Head of HSE

## 1. Purpose

This procedure defines mandatory controls for entry into any confined space — including pump pits, vessels (V-series), tanks (TK-series), columns (C-series), and below-grade pipelines — to prevent fatalities from toxic or oxygen-deficient atmospheres. It is mandatory under the Factories Act 1948 (Sec 36, 36A) and OISD-STD-116.

## 2. Scope

Applies to all contractors and employees entering any space classified as confined space in the Site Confined Space Register (HSE-REG-CS-2024).

## 3. Atmospheric Testing Requirements

The following limits must be confirmed by a certified gas tester before every entry:
  • Oxygen:          19.5 % – 23.5 %
  • Flammable Gas:   < 10 % LEL  (alarm at 5 % LEL)
  • Hydrogen Sulphide (H₂S): < 10 ppm TWA, < 15 ppm ceiling
  • Carbon Monoxide (CO): < 35 ppm TWA
Testing shall be continuous during entry using calibrated 4-gas detector.

## 4. Mandatory Controls

Before issuing a Confined Space Entry Permit (CSEP):
  1. Area Authority signs the CSEP; copies to Safety Dept and Shift Supervisor
  2. Equipment isolated, de-energised, and LOTO applied (see LOTO-SOP-015)
  3. Vessel purged and ventilated; forced air supply during entry
  4. Trained standby attendant posted outside — may not enter under any circumstance
  5. Rescue equipment (tripod, harness, SCBA) positioned at entry point
  6. Entrant wears personal 4-gas detector; evacuation if any alarm activates

## 5. SIMOPS Restrictions

Simultaneous Operations (SIMOPS) Policy per OISD-GDN-192:
  • No hot-work permit (HWP) within 15 metres of any open confined space
  • No pressure testing on adjacent lines while confined space is occupied
  • Inform SIMOPS coordinator 1 hour before entry; log in Daily SIMOPS Register

The 2024 site incident review (INC-2024-03) found that combined maintenance on P-101 and H-301 without a SIMOPS check preceded the most serious near-miss of the year. The investigation recommended mandatory SIMOPS briefing before every CSEP.

## 6. Rescue Plan

Non-entry rescue is the default (mechanical retrieval via tripod/harness). Entry rescue may only be initiated by a trained rescuer with SCBA; alert plant emergency response team (ERT) before entry rescue commences. Call emergency: Ext 100 (ERT), Ext 101 (Ambulance), Ext 102 (Fire Station).